## 3.4 Comparison with Analytical Solutions

Certain CFAST sub-models address phenomena that have analytical solutions, for example, one dimensional heat conduction through a solid or pressure increase in a sealed or slightly leaky compartment as a result of a fire or fan. The developers of CFAST use analytical solutions to test sub-models to verify the correctness of the coding of the model as part of the development. This section provides an overview of the verification testing conducted with each change of the model to verify the basic underlying principles of the model, the mass and energy balances.

For most of the examples presented in this chapter, the same basic geometry is used, a single 5 m x 5 m x 5 m compartment. Depending on the exact simulation, additional details will be added to verify individual model results.

As a simple test of the mass and energy balance, consider the base case with the addition of a fixed amount of air (injected through a mechanical vent). The vent is set to deliver 0.1 m3/s into the compartment for 10 s. After this, the vent is closed. Over the 10 s period, an additional 1 m3 of air at ambient temperature is added. Since CFAST, by default, closes vent over a 1 s time period, an additional 0.05 m3 is also added. With a compartment volume of 125 m3 at 20 C, the initial mass of air is 150.5 kg or 5.19 moles. With the added mass, the final mass of air in the compartment is 151.7 kg or 5.23 moles. From the CFAST output, the final temperature of the compartment rises slightly due to the pressure increase to 20.98 C. From the ideal gas law, the expected pressure rise can be calculated as

PiViNiRTi=PfVfNfRTf

Pf=PiNfTfNiTi

Pf=101300*5.23*294.25.19*293.2

=102491.2 Pa

or a pressure rise of 1192.41 Pa.  CFAST calculates a pressure rise of 1190.87 Pa, a relative difference of 0.129%.  Figure 3.2 shows the simulated conditions for this test.

Figure 3.2: Ambient conditions for test case with no fire and non-conducting surfaces, with a mechanical ventilation system injecting 0.1 m3/s of air into the compartment for 10 s.  CFAST verification file basic_mechvent.in.

A similar example can be tested when the temperature of the base case compartment is raised from 20 C to 25 C.  From the ideal gas law, the pressure rise can be calculated as

PiViNiRTi=PfVfNfRTf

Pf=Tf*PiTi

Pf=297.97*(101300273)

Pf=103018.3 Pa

or a pressure rise of 1718.3 Pa.  CFAST generates a pressure rise of 1714.16 Pa, or a relative difference of 0.09%. Figure 3.3 shows the conditions for this test.

Figure 3.3: Conditions for test case with no fire and 5/8 in Gypsum Board walls and ceiling, with the temperature raised from 20 C to 25 C.  CFAST verification file basic_tempequilib_in.

A model examining heat added to a system was demonstrated with test case containing a 10 s, 100 kW fire.  With non-conducting surfaces and no ventilation, the heat released from the fire can be determined with the heat transfer equation, Q=m*cp*∆T.  Using the CFAST generated final layer height of 3.26 m, the volume of the upper layer is calculated to be 43.451 m3 and the lower layer is 81.549 m3.  The proper densities and specific heats are obtained using the temperature of each layer.  With this information the air mass of the upper layer is 49.62 kg and the lower layer air mass is 97.44 kg.  The heat added to the system can be calculated as

Q=m*cp*∆T

Qupper=49.62*1.142*308.912-273.15

Qupper=786.246 kJ

Qlower=97.44*1.195*295.268-273.15

Qlower=207.251 kJ

Qtotal=993.497 kJ

or a total heat release of 993.497 kJ, or 99.35 kW, a 0.65% relative difference from the CFAST input of 100 kW. Figure 3.4 shows the simulated conditions for this test.

Figure 3.4: Heat release rate conditions for test case with a 10 s, 100 kW fire.  CFAST verification file 100kW_fire_in.

## 3.5 Simple Reference Cases

In addition to the comparisons of test cases with analytical solutions discussed in the previous section, we have included a number of simple test cases intended to demonstrate the impact of individual model features. While these are too complex to provide analytical solutions, many can be estimated with simpler engineering calculations. Together, these form a set of simulations that can be routinely run to verify continued consistency of the calculation results as the model is further developed to insure unintended side-effects of modifications to the model are prevented.

As in Section 3.4, for most of the examples presented in this chapter the same basic geometry is used, a single 5 m x 5 m x 5 m compartment. Unless otherwise detailed, the ‘base case’ has a single window centered on the front wall that is 2 m x 1 m and has 5/8 in Gypsum Board walls and ceiling. Depending on the exact simulation, additional details will be added to verify individual model results.

To begin, the simplest example is the base case compartment set at uniform ambient conditions with no ventilation, fires, or additional modeled features. With no added mass or energy to the system, conditions are expected to remain at the initial ambient. Figure 3.5 shows the simulated conditions for this simple test.

Figure 3.5: Ambient conditions for test case with no fire, no vents, and no additional features.  Initial conditions set to 20 C.  CFAST verification file Base.in.

The exterior temperature is raised to 25 C.  Over time, the interior compartment conditions are expected to equilibrate to those of the exterior.  Figure 3.6 shows the simulated conditions for the test case.

Figure 3.6: Test case conditions with no ventilation.  Exterior conditions set to 25 C.  CFAST verification file basic_tempequilib.in.

The same test is repeated with all surfaces turned off.  With the absence of conductive surfaces, compartment conditions are not expected to change. Figure 3.7 shows the simulated conditions for the test case.

Figure 3.7: Test case conditions with no ventilation and non-conducting surfaces.  Exterior conditions set to 25 C.  CFAST verification file basic_tempequilib_wallsoff.in.

The compartment surfaces are then turned back on, returning to the original base case.  With exterior conditions set to 25 C and a window present, conditions are expected to equilibrate to those of the exterior. Figure 3.8 shows the simulated conditions for the test case.

Figure 3.8: Test case conditions with exterior conditions set to 25 C.  CFAST verification file basic_tempequilib_window.in.

With the exterior temperature still set to 25 C, the elevation is raised to 1500 m, approximately the average elevation of Idaho.  Conditions are expected to equilibrate to those of the exterior. Figure 3.9 shows the simulated conditions for the test case.

Figure 3.9: Test case conditions with exterior conditions set to 25 C and elevation raised to 1500 m.  CFAST verification file basic_tempequilib_window_elevation.in.

With the exterior temperature still set to 25 C, the compartment dimensions are altered to 10 m x 10 m x 5 m.  Conditions are predicted to equilibrate to those of the exterior more slowly than that of the base case model. Figure 3.10 shows the simulated conditions for the test case.

Figure 3.10: Test case conditions with exterior conditions set to 25 C and compartment dimensions altered to 10 m x 10 m x 5m.  CFAST verification file basic_tempequilib_window_geometry.in.

With the exterior temperature still set to 25 C, the wind is raised to 10 m/s.  Temperature conditions are expected to equilibrate to those of the exterior, while the pressure will remain at a constant pressure greater than zero due to wind pressure. Figure 3.11 shows the simulated conditions for the test case.

Figure 3.11: Test case conditions with exterior conditions set to 25 C and wind speed raised to 10 m/s.  CFAST verification file basic_tempequilib_window_wind.in.

Figure 3.12: Comparison graphs displaying Upper Layer Temperature, Lower Layer Temperature, and Pressure for the temperature equilibrium case varying increased elevation, increased wind speed, and increased compartment size.

The exterior temperature is returned to 20 C, the base case input.  The interior pressure is then lowered from 101300 Pa to 100000 Pa, conditions are expected to equilibrate to those of the exterior.  Figure 3.13 shows the simulated conditions for the test case.

Figure 3.13: Test case conditions with interior pressure set to 100000 Pa.  CFAST verification file basic_pressure_vent.in.

With the interior pressure lowered from 101300 Pa to 100000 Pa and no window present, conditions are not expected to change.  Figure 3.14 shows the simulated conditions for the test case.

Figure 3.14: Test case conditions with interior pressure set to 100000 Pa and no ventilation.  CFAST verification file basic_pressure.in.

The interior pressure was then lowered from 101300 Pa to 100000 Pa and the surface turned off. Test case conditions are not expected to change; Figure 3.15 shows the simulated conditions.

Figure 3.15: Test case conditions with interior pressure set to 100000 Pa with surfaces off.  CFAST verification file basic_ pressure_wallsoff.in.

All ambient conditions are returned to the standard input.  A mechanical vent is added to the test case model with a flow rate of 0.1 m3/s.

Figure 3.16: Model with mechanical vent and non-inhibited drop off pressure of 2000 Pa versus model with mechanical vent and interfering drop off pressure of 200 Pa.

Two identical 5 m x 5 m x 5m compartments are set next to each other.  A 1 m2 mechanical vent is added on the front face of compartment one, the shared wall between compartment one and two, and the rear wall of compartment two.  The flow rate is set to 0.1 m3/s or 0.12 kg/s of air.  The mass flow through each of these vents is not expected to change because the flow rate in is constant.  Figure 3.17 shows the simulated conditions for the test case.

Figure 3.17: Side-by-side base case compartments with mechanical vents added from outside to compartment one, compartment one to compartment two, and compartment two to outside.  CFAST verification file basic_mechvent.in.

## Base Case Model with Fire

A fire was added to the base case.  The fire is positioned at X=2.5 m, Y=2.5 m, Z= 0, the material burning is methane and the max heat release rate if 1054 kW. All other fire inputs remain as the standard setting. Unless otherwise defined, the following reference cases contain this defined fire.

JPEG File: Mass Energy Balance>Screen Shots>Test Case HRR

JPEG File: Mass Energy Balance>Screen Shots>Test Case HRR Chart

Figure 3.18: Base case fire heat release rate curve and data table.

## Base Case Model with Fire, Ventilation

The following plots compare the base case model, including the defined fire, to an array of test cases that vary ventilation in the compartment.

Figure 3.19: Test case conditions with no ventilation versus the base case model.  CFAST verification file fire.in.

Figure 3.20: Test case conditions with two windows versus the base case model.  CFAST verification file fire_window_windowchange.in.

Figure 3.21: Test case conditions with compartment dimensions of 10 m x 10 m x 5 m, versus the base case model.  CFAST verification file fire_window_geochange.in.

Figure 3.22: Test case conditions with a ceiling vent of 1 m2 added, versus the base case model.  CFAST verification file fire_ ceiling.in.

Figure 3.23: Test case conditions with a ceiling vent increased to 2 m2 added, versus the previous test case with a 1 m2 ceiling vent.  CFAST verification files: fire_ ceiling_ventsize.in, fire_ ceiling.in.

Figure 3.24: Test case conditions with a 1 m2 ceiling vent and compartment dimensions increased to 10 m x 10 m x 5 m, versus the test case model with a 1 m2 ceiling vent.  CFAST verification files: fire_ ceiling_geochange.in, fire_ ceiling.in.

Figure 3.25: Test case conditions with a 1 m2 mechanical vent added and no window or ceiling ventilation, versus the base case model.  CFAST verification file fire_mechanical_vent.in.

With the mechanical vent still in place, the soot yield was raised from 0.00 to 0.50.  This test case was run and then compared to the subsequent test case in which the filter efficiency of the mechanical vent was set to 50%.  Trace species conditions can be viewed in Figure 3.26.

Figure 3.26: Test case conditions with mechanical vent operating at 0% and 50%. CFAST verification files: fire_filterefficiency.in and fire_filterefficiency_half.in.

## Base Case Model with Fire, Edited Fire

The following plots compare the base case model to an array of test cases that vary different features of the compartment fire.

Figure 3.27: Test case conditions with the maximum heat release rate doubled from 1054 kW to 2108 kW, versus the base case model.  CFAST verification file fire_HRRdoubled.in

The heat release rate curve was then altered to have the same area under the curve but a different shape.  The altered curve had a maximum heat release rate of 1333.3 kW and reached peak heat release rate at 450 s. The altered fire is shown in Figure 3.28.

JPEG File: Mass Energy Balance>Screen Shots>HRR2

JPEG File: Mass Energy Balance>Screen Shots>HRR2 Chart

Figure 3.28: Altered heat release rate triangle fire with maximum heat release rate of 1333.33 kW occurring at 450 s.

Figure 3.29: Test case conditions with the heat release rate curve changed from a t2 fire to a triangle shaped heat release rate, versus the base case model.  CFAST verification file fire_HRRdoubled.in.

Figure 3.30: Test case conditions with the burning material set to hexane instead of the standard methane, versus the base case model.  CFAST verification file fire_hexane.in.

Figure 3.31: Test case conditions with the burning material set to urethane instead of the standard methane, versus the base case model.  CFAST verification file fire_urethane.in.

Figure 3.32: Test case conditions with the burning material set to hardwood instead of the standard methane, versus the base case model.  CFAST verification file fire_hardwood.in.

Figure 3.33: Comparison graphs displaying the Upper Layer Temperature, Lower Layer Temperature, and Hot Gas Layer Height of the four tested materials: methane, hexane, urethane, and hardwood.

Figure 3.34: Comparison graphs displaying the Upper Layer Temperature, Lower Layer Temperature, and Hot Gas Layer Height of four test cases each with a different Gaseous Ignition Temperature.

Figure 3.35: Test case conditions with the soot yield raised to 0.500, versus the base case model.  CFAST verification file fire_sootyield.in.

Figure 3.36: Test case conditions with the CO yield raised to 0.500, versus the base case model.  CFAST verification file fire_CO_yield.in.

## Base Case Model with Fire, Sprinkler

The following plots compare the base case model to an array of test cases that vary the sprinkler feature of CFAST.    The sprinkler was added at X=2.5 m, Y=2.5 m, and Z=4.95 m.

Figure 3.37: Test case conditions with a sprinkler added to the compartment versus the base case model containing no sprinkler.  CFAST verification file fire_sprinkler.in.

The following test cases include a graph depicting the sensor activation time.  When these graphs rise from zero to one, this indicates the time that the sprinkler was activated.

Figure 3.38: Test case conditions with the sprinkler spray density doubled from 7x10-5 m/s to 1.4x10-4 m/s added to the compartment versus the test case containing the standard sprinkler.  CFAST verification file fire_sprinkler_density.in.

Figure 3.39: Test case conditions with the response time index (RTI) reduced to from 100 m1/2s1/2 to 50 m1/2s1/2, versus the test case containing the standard sprinkler.  CFAST verification files: fire_sprinkler_RTI.in, fire_sprinkler.in.

Figure 3.40: Test case conditions with the standard sprinkler and the maximum heat release rate of the fire doubled to 2108 kW, versus the test case containing the standard sprinkler.  CFAST verification files: fire_sprinkler_HRRdoubled.in, fire_sprinkler.in.

## Department of Energy Test Cases

In the CFAST Guidance Report, a series of test cases were modeled for the Department of Energy safety analysis.  In the report, these cases were run using CFAST Version (XX), they were then recreated in the current version of CFAST and compared.

Figure 3.41: Department of Energy test cases, CFAST Version (XX) versus CFAST Version 6.